School of Electrical and Computer Engineering

Embedded System Design - ECE 5721 - Fall 2022

Lab 1

Daniel Funke

September 20, 2022

Contents

1.	Downloading Software Tools	2

1.1	Description	2

1.1.1	Objective	2

1.2	Methods and Results	2

1.2.1	Install Keil MDK Software	2

2.	Blinky Example	3

2.1	Description	3

2.1.1	Objective	3

2.2	Methods and Results	3

2.2.1	Opening the Blinky Example	3

2.2.2	Building the Source Code	3

2.2.3	Loading and Running the Executable on the Target Hardware	4

2.2.4	Using the Debugger	5

3.	Creating a Project	6

3.1	Description	6

3.1.1	Objective	6

3.2	Methods and Results	6

3.2.1	Creating the Project and Writing the Code	6

3.2.2	Building and Running the Code	7

3.2.3	Using the Debugger to Confirm Functionality	7

4.	Appendix	9

4.1	Blinky Source Code	9

4.2	BlinkyNEW Source Code	11

5.	References	12

# Downloading Software Tools

## Description

We will be using the NXP FRDM-KL25Z development board as the target hardware for this lab. This board includes an Arm Cortex-M0+ microcontroller. To assist with the development of embedded applications for this target board, we will be utilizing Microcontroller Developer’s Kit (MDK), which includes the µVision Integrated Development Environment (IDE), provided by Keil, a subsidiary of Arm Holdings. The MDK provides the necessary files for configuring a project for the FRDM-KL25Z, while the µVision IDE provides the tools necessary to build, debug, and load executable code onto the target hardware.

### Objective

The purpose of this experiment is to install and configure the Keil MDK and µVision Integrated Development Environment (IDE) for use with the FRDM-KL25Z development board.

## Methods and Results

### Install Keil MDK Software

Software was downloaded from www.keil.com/mdk5/install and installed onto my local PC. The Keil MDK Software installation procedure described in Sections 4 through 6 of the Cortex-M0+ Lab document [1] was followed to install the necessary software and board support packages. This procedure was completed without error. Figure 1 shows the landing page for the µVision IDE with the Blinky example project opened to demonstrate successful completion of the installation process.

Figure 1: µVision IDE with Blinky example project opened

# Blinky Example

## Description

The “Blinky” example project exists as an embedded analog to the classic “Hello, World” example project utilized in virtually every computer programming. This example utilizes a microcontroller to drive an output pin to control the state of an LED. The Keil MDK, installed in the previous section, includes an instance of this example project to demonstrate the basic functionality of the development environment and the target hardware. This section of Lab 1 examines the included “Blinky” example.

### Objective

The purpose of this exercise is to gain familiarity with the Keil development environment and the NXP FRDM-KL25Z, including the building, debugging, and loading phases of embedded software development.

## Methods and Results

### Opening the Blinky Example

The Blinky example project was installed during Part 1 of this Lab exercise. To open the project, µVision was launched from Windows, then “Open Project” was selected from the “Project” dropdown menu. The Blinky project file was selected from File Explorer and loaded into µVision.

### Building the Source Code

The source files were compiled by selecting the “Rebuild” icon in µVision. An error was encountered at this point. The default compiler for this sample project, Arm Compiler 5, has been deprecated and is no longer included in the standard installation of the Keil MDK. This problem was remedied by selecting the “Options for target..” icon and navigating to the “Target” tab. From there, the ARM compiler was set to version 6, as shown in Figure 2. After updating the default compiler, the source code build without error.

Figure 2 Target options menu

### Loading and Running the Executable on the Target Hardware

The FRDM-KL25Z was connected to the host computer via a USB cable. Connectivity with the target hardware was confirmed by inspecting the Debugger Settings window found in the Target Options menu under the Debug tab. This procedure is described in Section 7 of [1]. Figure 3 shows the presence of an IDCODE under the SW Device field, indicating a successful connection to the target.

Figure 3 Debugger Settings

The executable image was transferred to the target hardware by selecting the Debug icon within µVision. Using the Debug option enables us to set breakpoints and examine the call stack, local variables, and memory contents. Once the executable was successfully transferred to the target board, the “Run” icon was selected to start execution. The board’s RBG LED immediately began to flash at a constant rate, cycling between red, green, and blue on successive illuminations.

### Using the Debugger

A breakpoint was inserted at Line 43 by selecting the left side column of the µVision text editor. This halted program execution at this location in the source code. Figure 4 shows the Debugger’s “Call Stack and Locals window,” which displays the value of the local variables belonging to the Thread_LED function (the function responsible for controlling the LED color and state). Table 1 displays the mapping between the color of the light emitted by the LED and its respective value in code represented by the led_num value.

Figure 4: Debugger Call Stack + Locals window

Table 1: LED colors and their respective value in the source code

| LED Color | led_num Value |
| --- | --- |
| Red | 0x00000000 |
| Green | 0x00000001 |
| Blue | 0x00000002 |

The source code for this example project is included in Appendix Section 4.1 . It is also included as a .c file with the lab submission.

# Creating a Project

## Description

Section 2 of this lab exercise provided some familiarity with the software tools and hardware platform by building, loading, and running an example project provided with the Keil MDK. This section, however, aimed to expand this basic understanding by instructing the user to create a project from scratch.

### Objective

The purpose of this exercise is to expand upon the basic understanding of the Keil MDK, provided in Section 2, by creating a new project from scratch.

## Methods and Results

### Creating the Project and Writing the Code

A new project named “BlinkyNEW” was created within µVision by following the instructions described in Section 24 of [1]. Once the project was launched, a .c file was created that contained a main function that incremented a counter value every loop, resetting the value once it reached 16. The code that implemented this functionality is shown in Section 4.2 . It is also included as a file attachment with this lab submission.

Figure 5: Flow diagram of “BlinkyNEW” project

### Building and Running the Code

The source code shown in Section 4.2 built and ran without error.

### Using the Debugger to Confirm Functionality

To confirm proper functionality of the source code, a break point was added to the line where the counter variable is incremented (line 13). Debugging mode was engaged and the “Run” button was select to run the code to this breakpoint. As expected, the initialized value incremented from 0 to 1 after executing this line. This result is shown in the pair of screen shots shown in Figure 6 and Figure 7.

Figure 6: Debugger Watch window displaying the initialized counter variable value of 0x00000000

Figure 7: Debugger Watch window displaying the incremented value of counter variable of 0x00000001 after a single iteration of the main while loop

# Appendix

## Blinky Source Code

/*----------------------------------------------------------------------------

* Name:    Blinky.c

* Purpose: LED Flasher

*----------------------------------------------------------------------------

* This file is part of the uVision/ARM development tools.

* This software may only be used under the terms of a valid, current,

* end user licence from KEIL for a compatible version of KEIL software

* development tools. Nothing else gives you the right to use this software.

*

* This software is supplied "AS IS" without warranties of any kind.

*

* Copyright (c) 2015 Keil - An ARM Company. All rights reserved.

*----------------------------------------------------------------------------*/

#include "MKL25Z4.h"                    // Device header

#include "cmsis_os.h"                   // ARM::CMSIS:RTOS:Keil RTX

#include "Board_LED.h"                  // ::Board Support:LED

/*----------------------------------------------------------------------------

*      Thread 1 'Thread_LED': Sample thread

*---------------------------------------------------------------------------*/

void Thread_LED (void const *argument);                             // thread function

osThreadId tid_Thread_LED;                                          // thread id

osThreadDef (Thread_LED, osPriorityNormal, 1, 0);                   // thread object

int Init_Thread_LED (void) {

tid_Thread_LED = osThreadCreate (osThread(Thread_LED), NULL);

if(!tid_Thread_LED) return(-1);

return(0);

}

void Thread_LED (void const *argument) {

uint32_t led_max    = LED_GetCount();

uint32_t led_num    = 0;

while (1) {

LED_On(led_num);                                                // Turn specified LED on

osSignalWait(0x0001, osWaitForever);

LED_Off(led_num);                                               // Turn specified LED off

osSignalWait(0x0001, osWaitForever);

led_num++;                                                      // Change LED number

if (led_num >= led_max) {

led_num = 0;                                                  // Restart with first LED

}

osThreadYield();                                                // suspend thread

}

}

/*----------------------------------------------------------------------------

* main: blink LED and check button state

*----------------------------------------------------------------------------*/

int main (void) {

osKernelInitialize ();                                            // initialize CMSIS-RTOS

SystemCoreClockUpdate();

LED_Initialize();                                                 // initalize LEDs

Init_Thread_LED();                                                // create LED thread

osKernelStart ();                                                 // start thread execution

for (;;) {                                                        // main must not be terminated!

osDelay(500);

osSignalSet(tid_Thread_LED, 0x0001);

}

}

## BlinkyNEW Source Code

- #include <MKL25Z4.h>

/* Static variable used for counter */

static unsigned int counter = 0;

int main(void)

{

SystemCoreClockUpdate();

/* Main loop */

while(1) {

/* Increment counter */

counter++;

/* Reset counter at 15 */

if (counter > 0x0F) {

counter = 0;

}

}

}

# References

| [1] | R. Boys, "NXP Kinetis L Series: Cortex-M0+ Lab Using the Freedom KL25Z," Fall 2017. [Online]. Available: https://moodle.oakland.edu/pluginfile.php/7958115/mod_resource/content/1/NXP%20Lab%20-30%20pages%20using%20ARM%20Keil%20MDK.pdf. [Accessed 2022]. |
| --- | --- |